REVIEW EXERCISE #7

| 1. | Where will you find detailed information regarding Provisions Concerning Passenger and Crew? |
| --- | --- |
| 2. | For each of the following items, indicate what undeclared dangerous goods may be present. |
| a) | Electrically powered equipment: |
| b) | Frozen fruit/vegetables/meat: |
| 3. | For each of the following substances in Limited Quantities, indicate the Packing Instruction and maximum quantity permitted in one outer package and how many outer packages are required. |

| Substance | Packing Instruction | Maximum Quantity Per Outer Package | Number of Outer Packages |
| --- | --- | --- | --- |
| 10 L Acetal |  |  |  |
| 50 kg Acridine |  |  |  |

| 4. | What is the maximum Gross Weight of a Limited Quantity package? |
| --- | --- |
|  | 30 kg. |

| 5. | For each of the following substances in Excepted Quantities, indicate the EQ Code, the maximum quantity permitted in one inner package, the maximum quantity permitted in one outer package, how many outer packages are required. |
| --- | --- |

| Substance | EQ Code | Maximum Quantity Per Inner Package | Maximum Quantity Per Outer Package | Number of Outer Packages |
| --- | --- | --- | --- | --- |
| 10 kg Barium |  |  |  |  |
| 5 kg Butane |  |  |  |  |

| 6. | For each of the following shipments, indicate any applicable Operator Variations. |
| --- | --- |

| Shipment Details | Applicable Operator Variations |
| --- | --- |
| a) | - L of 2-Chloropropane in UN 4G combination packages. - Hong Kong to Bangkok on EVA Airways passenger aircraft. |  |
| b) | 20 kg of Chromium nitrate in Limited Quantity combination packages. - Johannesburg to Frankfurt on Air Namibia passenger aircraft. |  |
| c) | 25 kg of Ferric nitrate in UN 4D combination packages. - Chicago to Paris on United Airlines passenger aircraft. |  |

| 7. | For the following shipment, is the portion of Cargo Air Waybill, the package and the Shipper’s Declaration below completed correctly? If not, why not? |
| --- | --- |
|  | - 10 L Gasoline, Limited Quantities, passenger aircraft. |
|  | AWB: |
|  | Package: |
|  | DGD: |

| Airport of Departure Buenos Aires |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| To SCL |  | By first carrier Aerolineas Argentinas |  |  | To |  | By | To |  | By |  |
| Airport of Destination Santiago |  |  |  |  |  |  |  |  |  |  |  |
| Handling Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dangerous goods as per attached Shipper’s Declaration. |  |  |  |  |  |  |  |  |  |  |  |  |  | SCI |
| Number of Pieces | Gross Weight |  | kg lb | Rate Class |  | Chargeable Weight |  |  | Rate / Charge |  | Total |  | Nature and Quantity of Goods |  |
| 10 | 15 |  | K |  |  |  |  |  |  |  |  |  | Samples of gasoline for test purposes. |  |

u     4G/X  10/S/11

n        XX/ABC123

Shipper:			Consignee:

Argentinas Chemicals SA	Laboratoria Chile SA

Av. Independencia 110	Av. Moron 1400

Buenos Aires, Argentina	Santiago, Chile

UN1203  Gasoline

| Shipper Argentinas Chemicals SA Av. Independencia 110 Buenos Aires, Argentina |  |  |  |  |  | Air Waybill No. 044 – 5556 6665 Page _1___ of 1___ Pages Shipper’s Reference Number __________________ |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Consignee Laboratoria Chile SA Av. Moron 1400 Santiago, Chile |  |  |  |  |  |  |  |  |  |  |
| Two completed and signed copies of this Declaration must be handed to the operator. |  |  |  |  |  | WARNING: Failure to comply in all respects with the applicable Dangerous Goods Regulations may be in breach of the applicable law, subject to legal penalties. |  |  |  |  |
| TRANSPORTATION DETAILS |  |  |  |  |  | This Declaration must not, in any circumstances, be |  |  |  |  |
| This shipment is within the limitations prescribed for: (delete non-applicable) PASSENGER CARGO AND CARGO AIRCRAFT AIRCRAFT ONLY |  | Airport of Departure: Buenos Aires |  |  |  | completed and/or signed by a consolidator, a forwarder or an IATA Cargo Agent. |  |  |  |  |
| Airport of Destination: Santiago |  |  |  |  |  | Shipment type (delete non-applicable): NON-RADIOACTIVE RADIOACTIVE |  |  |  |  |
| NATURE AND QUANTITY OF DANGEROUS GOODS |  |  |  |  |  |  |  |  |  |  |  |
| Dangerous Goods Identification |  |  |  |  |  |  |  |  |  |  |  |
| UN or ID No. |  | Proper Shipping Name |  | Class or Division (Subsidiary Risk) |  | Packing Group |  | Quantity and type of Packing | Packing Inst. |  | Author- ization |
| ###### UN 1203 |  | Gasoline |  | 3 |  | II |  | 10 Fibreboard boxes x 1 L | 341 |  | E2 |
| Additional Handling Information: |  |  |  |  |  |  |  |  |  |  |  |
| I hereby declare that the contents of this consignment are fully and accurately described above by the proper shipping name and are classified, packaged, marked and labelled, and are in all respects in the proper condition for transport by air according to the applicable International and National Government Regulations. I declare that all of the applicable air transport requirements have been met. |  |  |  | Name/Title of Signatory: Edwardo Montoya, Packer Place and Date: Buenos Aires, October 1, 2011 Signature: EMontoya |  |  |  |  |  |  |

| 8. | For the following shipment, is the portion of Cargo Air Waybill, the package and the Shipper’s Declaration below completed correctly? If not, why not? |
| --- | --- |
|  | - 25 kg Iodine, Limited Quantities, passenger aircraft. |
|  | AWB: |
|  | Package: |
|  | DGD |

| Airport of Departure Frankfurt |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| To FCO |  | By first carrier Lufthansa |  |  | To |  | By | To |  | By |  |
| Airport of Destination Rome |  |  |  |  |  |  |  |  |  |  |  |
| Handling Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dangerous goods as per attached Shipper’s Declaration. |  |  |  |  |  |  |  |  |  |  |  |  |  | SCI |
| Number of Pieces | Gross Weight |  | kg lb | Rate Class |  | Chargeable Weight |  |  | Rate / Charge |  | Total |  | Nature and Quantity of Goods |  |
| 5 | 35 |  | K |  |  |  |  |  |  |  |  |  | Iodine / UN3495 / Class 8 (6.1) / Packing Group III |  |

Y

Iodine  UN3495

Shipper:			      Consignee:

D-Chem AG			      Farmacia Corsi

1077 Industriepark		      Via Del Corsi 15

Frankfurt am Main, Germany	      Rome, Italy

| Shipper D-Chem AG 1077 Industriepark Frankfurt am Main, Germany |  |  |  |  |  | Air Waybill No. 020 – 6633 4422 Page 1 of 1 Pages Shipper’s Reference Number __________________ |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Consignee Farmacia Corsi Via del Corsi 15 Rome, Italy |  |  |  |  |  |  |  |  |  |  |
| Two completed and signed copies of this Declaration must be handed to the operator. |  |  |  |  |  | WARNING: Failure to comply in all respects with the applicable Dangerous Goods Regulations may be in breach of the applicable law, subject to legal penalties. |  |  |  |  |
| TRANSPORTATION DETAILS |  |  |  |  |  | This Declaration must not, in any circumstances, be |  |  |  |  |
| This shipment is within the limitations prescribed for: (delete non-applicable) PASSENGER CARGO AND CARGO AIRCRAFT AIRCRAFT ONLY |  | Airport of Departure: Rome |  |  |  | completed and/or signed by a consolidator, a forwarder or an IATA Cargo Agent. |  |  |  |  |
| Airport of Destination: Frankfurt |  |  |  |  |  | Shipment type (delete non-applicable): NON-RADIOACTIVE RADIOACTIVE |  |  |  |  |
| NATURE AND QUANTITY OF DANGEROUS GOODS |  |  |  |  |  |  |  |  |  |  |  |
| Dangerous Goods Identification |  |  |  |  |  |  |  |  |  |  |  |
| UN or ID No. |  | Proper Shipping Name |  | Class or Division (Subsidiary Risk) |  | Packing Group |  | Quantity and type of Packing | Packing Inst. |  | Author- ization |
| ###### ID 3495 |  | Iodine |  | 8 |  | III |  | 5 Fibre drums (1G) x 5 kg | Y845 |  | - |
| Additional Handling Information: |  |  |  |  |  |  |  |  |  |  |  |
| I hereby declare that the contents of this consignment are fully and accurately described above by the proper shipping name and are classified, packaged, marked and labelled, and are in all respects in the proper condition for transport by air according to the applicable International and National Government Regulations. I declare that all of the applicable air transport requirements have been met. |  |  |  | Name/Title of Signatory: Joop von Burke, Manager Place and Date: Frankfurt am Main, 1OCT2011 Signature: JoopvonBurke |  |  |  |  |  |  |

CASE STUDY #5

|  | For the following shipment, is the portion of Cargo Air Waybill, the package and the Shipper’s Declaration below completed correctly? If not, why not? |
| --- | --- |
|  | AWB: |
|  | Package: |
|  | DGD: |

| Airport of Departure Tokyo |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| To YVR |  | By first carrier Air Canada |  |  | To |  | By | To |  | By |  |
| Airport of Destination Vancouver |  |  |  |  |  |  |  |  |  |  |  |
| Handling Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dangerous goods as per attached Shipper’s Declaration. |  |  |  |  |  |  |  |  |  |  |  |  |  | SCI |
| Number of Pieces | Gross Weight |  | kg lb | Rate Class |  | Chargeable Weight |  |  | Rate / Charge |  | Total |  | Nature and Quantity of Goods |  |
| 4 | 260 |  | K |  |  |  |  |  |  |  |  |  | Liquid chemicals (Isobutanol, UN1212) |  |

u     4G/Z  75/IP/10

n        J/1-36-33

Isobutanol  UN1212

Shipper:			consignee:

Nippon Petro Co. Ltd.	west industries ltd.

2-44-6 Sinkawa		2257 coast rd.

tokyo, japan		vancouver, canada

| Shipper nippon petro co. ltd. 2-44-6 sinkawa tokyo, japan |  |  |  |  |  | Air Waybill No. 014 – 0011 1100 Page 1 of 1 Pages Shipper’s Reference Number __________________ |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Consignee west industries ltd. 2257 coast rd. vancouver, canada |  |  |  |  |  |  |  |  |  |  |
| Two completed and signed copies of this Declaration must be handed to the operator. |  |  |  |  |  | WARNING: Failure to comply in all respects with the applicable Dangerous Goods Regulations may be in breach of the applicable law, subject to legal penalties. |  |  |  |  |
| TRANSPORTATION DETAILS |  |  |  |  |  | This Declaration must not, in any circumstances, be |  |  |  |  |
| This shipment is within the limitations prescribed for: (delete non-applicable) PASSENGER CARGO AND CARGO AIRCRAFT AIRCRAFT ONLY |  | Airport of Departure: tokyo |  |  |  | completed and/or signed by a consolidator, a forwarder or an IATA Cargo Agent. |  |  |  |  |
| Airport of Destination: vancouver |  |  |  |  |  | Shipment type (delete non-applicable): NON-RADIOACTIVE RADIOACTIVE |  |  |  |  |
| NATURE AND QUANTITY OF DANGEROUS GOODS |  |  |  |  |  |  |  |  |  |  |  |
| Dangerous Goods Identification |  |  |  |  |  |  |  |  |  |  |  |
| UN or ID No. |  | Proper Shipping Name |  | Class or Division (Subsidiary Risk) |  | Packing Group |  | Quantity and type of Packing | Packing Inst. |  | Author- ization |
| ###### un 1212 |  | isobutanol |  | 3 |  | III |  | 4 Fibreboard boxes (4g) x 65 kg | 355 |  | - |
| Additional Handling Information: |  |  |  |  |  |  |  |  |  |  |  |
| I hereby declare that the contents of this consignment are fully and accurately described above by the proper shipping name and are classified, packaged, marked and labelled, and are in all respects in the proper condition for transport by air according to the applicable International and National Government Regulations. I declare that all of the applicable air transport requirements have been met. |  |  |  | Name/Title of Signatory: Yo Akashi, Export Clerk Place and Date: tokyo, OCT. 1, 2011 Signature: Yori Horishita |  |  |  |  |  |  |

CASE STUDY #6

|  | For the following shipment, is the portion of Cargo Air Waybill, the package and the Shipper’s Declaration below completed correctly? If not, why not? |
| --- | --- |
|  | AWB: |
|  | Package: |
|  | DGD: |

| Airport of Departure Toronto |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| To AMS |  | By first carrier KLM |  |  | To |  | By | To |  | By |  |
| Airport of Destination Amsterdam |  |  |  |  |  |  |  |  |  |  |  |
| Handling Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dangerous goods as per attached Shipper’s Declaration. |  |  |  |  |  |  |  |  |  |  |  |  |  | SCI |
| Number of Pieces | Gross Weight |  | kg lb | Rate Class |  | Chargeable Weight |  |  | Rate / Charge |  | Total |  | Nature and Quantity of Goods |  |
| 2 1 | 68 33 |  | K K |  |  |  |  |  |  |  |  |  | Allyl bromide / UN1099 Allyltrichlorosilane, stabilized / UN1724 Overpacked into one wooden box |  |

Shipper:			Consignee:

DB Chemicals Ltd.		Shell Laboratories Plc

54507 Highway 50		Airpark Complex 5-A

Bolton, Canada			Schiphol, Netherlands

UN1099 Allyl bromide	UN1724 Allyltrichlorosilane, stabilized

OVERPACK

USED

| Shipper DB Chemicals Ltd. 54507 Highway 50 Bolton, Canada |  |  |  |  |  | Air Waybill No. 074 – 4567 5678 Page 1 of 1 Pages Shipper’s Reference Number __________________ |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Consignee Shell Laboratories Plc Airpark Complex 5-A Schiphol, Netherlands |  |  |  |  |  |  |  |  |  |  |
| Two completed and signed copies of this Declaration must be handed to the operator. |  |  |  |  |  | WARNING: Failure to comply in all respects with the applicable Dangerous Goods Regulations may be in breach of the applicable law, subject to legal penalties. |  |  |  |  |
| TRANSPORTATION DETAILS |  |  |  |  |  | This Declaration must not, in any circumstances, be |  |  |  |  |
| This shipment is within the limitations prescribed for: (delete non-applicable) PASSENGER CARGO AND CARGO AIRCRAFT AIRCRAFT ONLY |  | Airport of Departure: Toronto |  |  |  | completed and/or signed by a consolidator, a forwarder or an IATA Cargo Agent. |  |  |  |  |
| Airport of Destination: Amsterdam |  |  |  |  |  | Shipment type (delete non-applicable): NON-RADIOACTIVE RADIOACTIVE |  |  |  |  |
| NATURE AND QUANTITY OF DANGEROUS GOODS |  |  |  |  |  |  |  |  |  |  |  |
| Dangerous Goods Identification |  |  |  |  |  |  |  |  |  |  |  |
| UN or ID No. |  | Proper Shipping Name |  | Class or Division (Subsidiary Risk) |  | Packing Group |  | Quantity and type of Packing | Packing Inst. |  | Author- ization |
| ###### UN 1099 UN 1724 |  | Allyl bromide Allyltrichlorosilane |  | 6.1 (3) 8 (3) |  | I II |  | 2 Steel drums (1A1) x 30 L 1 Steel drum (1A1) x 30 L Overpack | 361 876 |  | - - |
| Additional Handling Information: 24-Hour Number: +1-111-111-1111 |  |  |  |  |  |  |  |  |  |  |  |
| I hereby declare that the contents of this consignment are fully and accurately described above by the proper shipping name and are classified, packaged, marked and labelled, and are in all respects in the proper condition for transport by air according to the applicable International and National Government Regulations. I declare that all of the applicable air transport requirements have been met. |  |  |  | Name/Title of Signatory: Dave Brown, Chemist Place and Date: Bolton, October 1, 2011 Signature: DBrown |  |  |  |  |  |  |

CASE STUDY #7

|  | For the following shipment, is the portion of Cargo Air Waybill, the package and the Shipper’s Declaration below completed correctly? If not, why not? |
| --- | --- |
|  | AWB: |
|  | Package: |
|  | DGD: |

| Airport of Departure Sydney |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| To SIN |  | By first carrier Qantas |  |  | To |  | By | To |  | By |  |
| Airport of Destination Singapore |  |  |  |  |  |  |  |  |  |  |  |
| Handling Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dangerous goods as per attached Shipper’s Declaration |  |  |  |  |  |  |  |  |  |  |  |  |  | SCI |
| Number of Pieces | Gross Weight |  | kg lb | Rate Class |  | Chargeable Weight |  |  | Rate / Charge |  | Total |  | Nature and Quantity of Goods |  |
| 50 | 75 |  | K |  |  |  |  |  |  |  |  |  | Liquid chemicals in Limited Quantities |  |

UN1213 Iso acetate

Shipper:			Consignee:

Oz Chemicals Pty Ltd	Singapore Industrial Pte Ltd

244 Rose Hill Station	7 South Bridge Rd

Sydney, Australia		Singapore

Limited Quantity

| Shipper Oz Chemicals Pty Ltd 244 Rose Hill Station Sydney, Australia |  |  |  |  |  | Air Waybill No. 081 – 1212 3434 Page 1 of 1 Pages Shipper’s Reference Number __________________ |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Consignee Singapore Industrial Pte Ltd 7 South Bridge Rd Singapore |  |  |  |  |  |  |  |  |  |  |
| Two completed and signed copies of this Declaration must be handed to the operator. |  |  |  |  |  | WARNING: Failure to comply in all respects with the applicable Dangerous Goods Regulations may be in breach of the applicable law, subject to legal penalties. |  |  |  |  |
| TRANSPORTATION DETAILS |  |  |  |  |  | This Declaration must not, in any circumstances, be |  |  |  |  |
| This shipment is within the limitations prescribed for: (delete non-applicable) PASSENGER CARGO AND CARGO AIRCRAFT AIRCRAFT ONLY |  | Airport of Departure: Sydney |  |  |  | completed and/or signed by a consolidator, a forwarder or an IATA Cargo Agent. |  |  |  |  |
| Airport of Destination: Singapore |  |  |  |  |  | Shipment type (delete non-applicable): NON-RADIOACTIVE RADIOACTIVE |  |  |  |  |
| NATURE AND QUANTITY OF DANGEROUS GOODS |  |  |  |  |  |  |  |  |  |  |  |
| Dangerous Goods Identification |  |  |  |  |  |  |  |  |  |  |  |
| UN or ID No. |  | Proper Shipping Name |  | Class or Division (Subsidiary Risk) |  | Packing Group |  | Quantity and type of Packing | Packing Inst. |  | Author- ization |
| ###### un 1213 |  | isobutyl acetate |  | 3 |  | II |  | 50 Fibreboard boxes x 1 L | 341 |  | Ltd qty |
| Additional Handling Information: |  |  |  |  |  |  |  |  |  |  |  |
| I hereby declare that the contents of this consignment are fully and accurately described above by the proper shipping name and are classified, packaged, marked and labelled, and are in all respects in the proper condition for transport by air according to the applicable International and National Government Regulations. I declare that all of the applicable air transport requirements have been met. |  |  |  | Name/Title of Signatory: Michael Murphy, Clerk Place and Date: sydney, 1OCT2011 Signature: MMurphy |  |  |  |  |  |  |